October 9, 2002

Mr. Ali Abedali

2309 Elizabeth Street

Metairie, Louisiana  70003

RE:
Lot 11, Sq. 5, Pontchartrain Shores


Jefferson Parish, Louisiana

Dear Mr. Abedali,

I have investigated the beams as you requested.  The beam, as designed, provided the following deflections:


W14 x 48 @ 20 ft. long


Using a 150 lb./ft2 loading d = .57”


W14 x 61


Using a 150 lb./ft2 loading d = 1.6”

Equivalent 12” beams would be:

You may use:


W12 x 58 to replace the W14 x 48


The calculated deflection with the same loading = .558”

You may use:


W12 x 79 in place of the W14 x 61


The calculated deflection with the same loading = 1.54”

I also investigated the pilings shown on the plans.  Using the class 5, 40 ft. piles shown on the plans, and assuming that you get 5 tons capacity each, you could reduce the number of piles by about 40.  A test pile should be driven to confirm that the pile will support 5 tons.

I also recommend that you use a cable slab for this project.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

