554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

February 22, 2023

Pam Pucket

Pucket Team

1300 Gause Blvd c1, Slidell, LA 70458

RE:

1066-1068 Audubon St.

Slidell, La. 70460

Construction:

One-story, wood frame, brick veneer, terrazzo flooring, with a composition shingle roof on a conventional foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to cracks in the terrazzo flooring.

Findings:

Upon inspection, a zip level was used to verify the floor height conditions.  The level was set up at the front of the home in the front foyer. The slopes range in the home from 0.0” to -0.7” and are within the limits of elevation differences.

It was also noted that there are small hairline cracks in the terrazzo flooring on both sides of the units but are within limits along with some previous sheetrock repairs in the ceilings.

An overall visual inspection of the exterior of the home was conducted and noted on the right side of the home under the window, there are horizontal and vertical cracks in the brick veneer. Where the brick veneer wall sits on top of the foundation it was noted to have cracked in several areas and have an exposed footing.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

Recommendation:

It is our opinion that the structural integrity of this home is currently sound, but the foundation will expand and contract with the ground movement and any standing water.  It is recommended when there is more than a 3” elevation difference (tripping hazard) to have the home stabilized with screw pilings.

Items to be addressed are as follows:

- Add gutters and downspouts and tie into subsurface drainage on all downspouts that are needed to allow for proper drainage of rainwater away from the foundation.

- Seal the cracks in the brick veneer and foundation with a non-shrinking grout, to prevent insects from entering the home.

See attachments;

Elevations

Pictures

Sincerely,

Brian Mistich, P.E.