554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

June 11, 2024

Atef Afkarian

3911 Kent Street

Slidell, La. 70458

Construction

One-story, wood frame, brick veneer, with a composition shingle roof on a conventional foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to an inspection that noted cracks in the brick veneer and uneven flooring.

Findings:

Upon inspection of the interior of the home, it was noted that the rear of the home was added onto sometime in the past. There is a smooth elevation difference between the existing home and the addition. The elevation difference is not a tripping hazard and less than a couple of inches in twelve feet.

It was also noted that there are small hairline cracks in the exterior brick veneer in the rear of the home of this addition under the window as noted in the attached pictures. Also, where the new foundation attaches to the existing foundation there is a hairline separation.

An overall visual inspection of the exterior brick veneer of the home was conducted and noted on the right side of the home that there was some separation in the brick facade that has been repointed and sealed off.  This wall area also has mold growing on it from rain water running off the roof and splashing back on the wall.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

Recommendation:

It is our opinion that the structural integrity of this home is currently sound, but the foundation will expand and contract with the ground movement and any standing water.  It is recommended when there is more than a 3” elevation difference (tripping hazard) to have the home stabilized with screw pilings.

Items to be addressed are as follows:

- Add gutters and downspouts and tie into subsurface drainage on all downspouts that are needed to allow for proper drainage of rainwater away from the foundation.

- Seal the cracks in the brick veneer and foundation with a non-shrinking grout, to prevent insects from entering the home.

See attachments;

Pictures

Sincerely,

Brian Mistich, P.E.