Tangipahoa Landfill Oxidation Pond Force Main

57510 Hano Rd 
Independence, LA 70443 


SECTION 311000

SITE CLEARING
Copyright 2001 (R 05/03) by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.
PART 1 -  GENERAL

1.1 RELATED DOCUMENTS
A. Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.
1.2 SUMMARY
A. This Section includes the following:

Adjust list below to suit Project.

1. Protecting existing trees, shrubs, groundcovers, plants and grass to remain.

2. Removing existing trees, shrubs, groundcovers, plants and grass.

3. Clearing and grubbing.

4. Stripping and stockpiling topsoil.

5. Removing above- and below-grade site improvements.

6. Disconnecting, capping or sealing, and removing site utilities.

7. Temporary erosion and sedimentation control measures.
1.3 DEFINITIONS
Retain terms that remain after this Section has been edited.

Revise first paragraph below to define "topsoil" according to local practices if required.  Topsoil defined below is raw material that may need screening and amending before meeting planting soil mix requirements.

A. Topsoil:  Natural or cultivated surface-soil layer containing organic matter and sand, silt, and clay particles; friable, pervious, and black or a darker shade of brown, gray, or red than underlying subsoil; reasonably free of subsoil, clay lumps, gravel, and other objects more than 2 inches  in diameter; and free of subsoil and weeds, roots, toxic materials, or other non-soil materials.

B. Tree Protection Zone:  Area surrounding individual trees or groups of trees to be protected during construction, and defined by the drip line of individual trees or the perimeter drip line of groups of trees, unless otherwise indicated.
1.4 MATERIAL OWNERSHIP
A. Except for stripped topsoil or other materials indicated to remain Owner's property, cleared materials shall become Contractor's property and shall be removed from Project site.
1.5 SUBMITTALS
A. Photographs or videotape, sufficiently detailed, of existing conditions of trees and plantings, adjoining construction, and site improvements that might be misconstrued as damage caused by site clearing.

Revise paragraph below to suit Project.

B. Record drawings, according to Division 1, identifying and accurately locating capped utilities and other subsurface structural, electrical, and mechanical conditions.
1.6 QUALITY ASSURANCE
Delete this Article if Work of this Section is not extensive or complex enough to justify a preinstallation conference.  If retaining, coordinate with Division 1.

A. Preinstallation Conference:  Conduct conference at Project site to comply with requirements in Division 1 Section "Project Management and Coordination."
1.7 PROJECT CONDITIONS
A. Traffic:  Minimize interference with adjoining roads, streets, walks, and other adjacent occupied or used facilities during site-clearing operations.

1. Do not close or obstruct streets, walks, or other adjacent occupied or used facilities without permission from Owner and authorities having jurisdiction.

2. Provide alternate routes around closed or obstructed traffic ways if required by authorities having jurisdiction.

Delete paragraph and subparagraph below if not required.  Coordinate with Owner.

Delete first paragraph below if not applicable.  Indicate items to be salvaged on Drawings or in schedules.

B. Salvable Improvements:  Carefully remove items indicated to be salvaged and store on Owner's premises where indicated.

C. Utility Locator Service:  Notify utility locator service for area where Project is located before site clearing.

D. Do not commence site clearing operations until temporary erosion and sedimentation control measures are in place.
PART 2 -  PRODUCTS
Retain "(Not Applicable)" above and delete "Soil Materials" Article below when soil backfill is not required in below-grade areas after demolition.  Add other materials to suit Project.
2.1 SOIL MATERIALS
A. Satisfactory Soil Materials:  Requirements for satisfactory soil materials are specified in Division 2 Section "Earthwork."

1. Obtain approved borrow soil materials off-site when satisfactory soil materials are not available on-site.
PART 3 -  EXECUTION

3.1 PREPARATION
A. Protect and maintain benchmarks and survey control points from disturbance during construction.

B. Locate and clearly flag trees and vegetation to remain or to be relocated.

C. Protect existing site improvements to remain from damage during construction.

1. Restore damaged improvements to their original condition, as acceptable to Owner.
3.2 TEMPORARY EROSION AND SEDIMENTATION CONTROL
Delete this Article if erosion and sedimentation control measures are included in Division 1 "Temporary Facilities and Controls."

Add specific procedures or installation requirements for temporary soil protection and stabilization, erosion controls, stormwater runoff controls, and sedimentation controls.  If particular products are required, add product standards and descriptions to Part 2.  Select second option in first paragraph below to comply with LEED Prerequisite SS 1 if sediment and erosion control plan is included in the Contract Documents; select third option if Contractor is required to provide sediment and erosion control plan to comply with LEED Prerequisite SS 1.

A. Provide temporary erosion and sedimentation control measures to prevent soil erosion and discharge of soil-bearing water runoff or airborne dust to adjacent properties and walkways, according to requirements of authorities having jurisdiction.

B. Inspect, repair, and maintain erosion and sedimentation control measures during construction until permanent vegetation has been established.

C. Remove erosion and sedimentation controls and restore and stabilize areas disturbed during removal.
3.3 TREE PROTECTION
Delete this Article if not required or if requirements are specified in Division 2 Section "Tree Protection and Trimming."  If retaining, Drawings should show tree protection zones.

A. Erect and maintain temporary orange fencing around tree protection zones before starting site clearing.  Remove fence when construction is complete.

1. Do not store construction materials, debris, or excavated material within fenced area.

2. Do not permit vehicles, equipment, or foot traffic within fenced area.

3. Maintain fenced area free of weeds and trash.

B. Do not excavate within tree protection zones, unless otherwise indicated.

Retain paragraph below if construction excavation within drip line of trees is unavoidable.  Most roots of trees and shrubs occur in top 18 inches (450 mm) of soil.

C. Where excavation for new construction is required within tree protection zones, hand clear and excavate to minimize damage to root systems.  Use narrow-tine spading forks, comb soil to expose roots, and cleanly cut roots as close to excavation as possible.

Retain applicable subparagraphs below.

1. Cover exposed roots with burlap and water regularly.

2. Temporarily support and protect roots from damage until they are permanently redirected and covered with soil.

3. Coat cut faces of roots more than 1-1/2 inches in diameter with an emulsified asphalt or other approved coating formulated for use on damaged plant tissues.

4. Backfill with soil as soon as possible.

D. Repair or replace trees and vegetation indicated to remain that are damaged by construction operations, in a manner approved by Architect.

1. Employ an arborist, licensed in jurisdiction where Project is located, to submit details of proposed repairs and to repair damage to trees and shrubs.

Revise subparagraph above and below to suit Project.

2. Replace trees that cannot be repaired and restored to full-growth status, as determined by Architect.
3.4 UTILITIES
Retain paragraph and subparagraph below if Owner arranges this work.

A. Owner will arrange for disconnecting and sealing indicated utilities that serve existing structures before site clearing, when requested by Contractor.

1. Verify that utilities have been disconnected and capped before proceeding with site clearing.

Delete paragraph and subparagraph above and retain paragraph and applicable subparagraph below if Contractor arranges or performs this work.

B. Locate, identify, disconnect, and seal or cap off utilities indicated to be removed.

1. Arrange with utility companies to shut off indicated utilities.

Retain subparagraph above or below.

2. Owner will arrange to shut off indicated utilities when requested by Contractor.

Delete first paragraph below if no existing utilities.  Coordinate with requirements specified in Division 1 Section "Temporary Facilities and Controls" for temporary utilities.

C. Existing Utilities:  Do not interrupt utilities serving facilities occupied by Owner or others unless permitted under the following conditions and then only after arranging to provide temporary utility services according to requirements indicated:

Edit first subparagraph below to suit Project.

1. Notify Architect not less than two days in advance of proposed utility interruptions.

2. Do not proceed with utility interruptions without Architect's written permission.

D. Excavate for and remove underground utilities indicated to be removed.

Delete paragraph above or below.  If retaining above, coordinate removal of utilities with provisions for excavation, backfill, and compaction in Division 2 Section "Earthwork"; if retaining below, coordinate with applicable Sections in Division 2.

E. Removal of underground utilities is included in Division 2 Sections covering site utilities.
3.5 CLEARING AND GRUBBING
A. Remove obstructions, trees, shrubs, grass, and other vegetation to permit installation of new construction.

1. Do not remove trees, shrubs, and other vegetation indicated to remain or to be relocated.

2. Cut minor roots and branches of trees indicated to remain in a clean and careful manner where such roots and branches obstruct installation of new construction.

3. Grind stumps and remove roots, obstructions, and debris extending to a depth of 18 inches below exposed subgrade.

4. Use only hand methods for grubbing within tree protection zone.

5. Chip removed tree branches and dispose of off-site.

Delete paragraph and subparagraph below if not required.  Coordinate with Division 2 Section "Earthwork."

B. Fill depressions caused by clearing and grubbing operations with satisfactory soil material unless further excavation or earthwork is indicated.

1. Place fill material in horizontal layers not exceeding a loose depth of 8 inches, and compact each layer to a density equal to adjacent original ground.
3.6 TOPSOIL STRIPPING
A. Remove sod and grass before stripping topsoil.

B. Strip topsoil to whatever depths are encountered in a manner to prevent intermingling with underlying subsoil or other waste materials.

Revise subparagraph below if other materials are known to be mixed into unsuitable surface soil.

1. Remove subsoil and nonsoil materials from topsoil, including trash, debris, weeds, roots, and other waste materials.

Revise paragraph and subparagraphs below if topsoil is to be removed from site.

C. Stockpile topsoil materials away from edge of excavations without intermixing with subsoil.  Grade and shape stockpiles to drain surface water.  Cover to prevent windblown dust.

1. Limit height of topsoil stockpiles to 72 inches.
2. Do not stockpile topsoil within tree protection zones.

3. Dispose of excess topsoil as specified for waste material disposal.

Select subparagraph above or below.

4. Stockpile surplus topsoil to allow for re-spreading deeper topsoil.
3.7 SITE IMPROVEMENTS
Revise first paragraph below and add specific items to suit Project.

A. Remove existing above- and below-grade improvements as indicated and as necessary to facilitate new construction.

B. Remove slabs, paving, curbs, gutters, and aggregate base as indicated.

1. Unless existing full-depth joints coincide with line of demolition, neatly saw-cut length of existing pavement to remain before removing existing pavement.  Saw-cut faces vertically.

2. Paint cut ends of steel reinforcement in concrete to remain to prevent corrosion.
3.8 DISPOSAL
A. Disposal:  Remove surplus soil material, unsuitable topsoil, obstructions, demolished materials, and waste materials including trash and debris, and legally dispose of them off Owner's property.

Retain subparagraph below where recycling programs exist and recycling facilities can accept materials such as concrete or asphalt paving.

1. Separate recyclable materials produced during site clearing from other non-recyclable materials.  Store or stockpile without intermixing with other materials and transport them to recycling facilities.
END OF SECTION
SITE CLEARING

311000 - 6

