ST. TAMMANY FIRE PROTECTION DISTRICT NO. 1
NEW TRAINING FACILITY
34780 S. RANGE ROAD 

SLIDELL LA  70460


SECTION 015000
TEMPORARY FACILITIES AND CONTROLS
Copyright 2002, The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions and other Division 1 Specification Sections, apply to this Section.

1.2 SUMMARY

A. This Section includes requirements for temporary utilities, support facilities, and security and protection facilities.

1.3 DEFINITIONS

A. Permanent Enclosure:  As determined by Architect, permanent or temporary roofing is complete, insulated, and weathertight; exterior walls are insulated and weathertight; and all openings are closed with permanent construction or substantial temporary closures.

1.4 USE CHARGES

Retain first paragraph below to prevent extra cost charges to Owner.

A. General:  Cost or use charges for temporary facilities shall be included in the Contract Sum.  Allow other entities to use temporary services and facilities without cost, including, but not limited to, Owner's construction forces, Architect, testing agencies, and authorities having jurisdiction.

If needed, insert use-charge requirements for other utilities needed for construction operations.

1.5 SUBMITTALS

A. Site Plan:  Show temporary facilities, utility hookups, staging areas, and parking areas for construction personnel.

1.6 QUALITY ASSURANCE

A. Electric Service:  Comply with NECA, NEMA, and UL standards and regulations for temporary electric service.  Install service to comply with NFPA 70.

B. Tests and Inspections:  Arrange for authorities having jurisdiction to test and inspect each temporary utility before use.  Obtain required certifications and permits.

1.7 PROJECT CONDITIONS

Retain this Article to impose responsibility for maintenance and protection of permanent services and facilities used to provide temporary service and facility.

A. Temporary Use of Permanent Facilities:  Installer of each permanent service shall assume responsibility for operation, maintenance, and protection of each permanent service during its use as a construction facility before Owner's acceptance, regardless of previously assigned responsibilities.

PART 2 -  PRODUCTS

2.1 MATERIALS

A. Pavement:  Comply with Division 2 Section "Cement Concrete Pavement."

Delete first two paragraphs below if wood fence in third paragraph is used or covered walkway is used.  For more security, increase fence height to 8 feet (2.4 m) and revise size of posts.  If appearance is important for highly visible projects, revise paragraphs to require new materials; otherwise, Contractor is likely to provide reused material.  Delete all three paragraphs if fence material is Contractor's option.

B. Chain-Link Fencing:  Minimum 2-inch , 0.148-inch-thick, galvanized steel, chain-link fabric fencing; minimum 6 feet  high with galvanized steel pipe posts; minimum 2-3/8-inch-  OD line posts and 2-7/8-inch-  OD corner and pull posts, with 1-5/8-inch- OD top rails.

C. Portable Chain-Link Fencing:  Minimum 2-inch , 9-gage, galvanized steel, chain-link fabric fencing; minimum 6 feet  high with galvanized steel pipe posts; minimum 2-3/8-inch-  OD line posts and 2-7/8-inch-  OD corner and pull posts, with 1-5/8-inch-  OD top and bottom rails.  Provide concrete or galvanized steel bases for supporting posts.

If Sections in paragraph below are not included in Project, add requirements here.

D. Lumber and Plywood:  Comply with requirements in Division 6 Section “Rough Carpentry."

Retain paragraph below if gypsum board temporary partitions for enclosure or dust control are specified.

2.2 TEMPORARY FACILITIES

A. Drinking water and portable toilet facility.

Coordinate paragraph and subparagraph below with Owner for use of existing building for storage and protection of materials to be incorporated into Project.

B. Storage and Fabrication Sheds:  Provide sheds sized, furnished, and equipped to accommodate materials and equipment for construction operations.

A. Store combustible materials apart from building.

2.3 EQUIPMENT

A. Fire Extinguishers:  Portable, UL rated; with class and extinguishing agent as required by locations and classes of fire exposures.

Revise paragraph and subparagraphs below to suit Project.  Liquid-propane-gas or fuel-oil heaters are commonly used.  Steam or hot-water heaters, gas-fired space heaters, or electric unit heaters are also used.

PART 3 -  EXECUTION

3.1 INSTALLATION, GENERAL

A. Locate facilities where they will serve Project adequately and result in minimum interference with performance of the Work.  Relocate and modify facilities as required by progress of the Work.

Retain subparagraph below for greenfield sites if reduced site disturbance is required for LEED-NC Credit SS 5.1.

A. Locate facilities to limit site disturbance as specified in Division 1 Section "Summary."

B. Provide each facility ready for use when needed to avoid delay.  Do not remove until facilities are no longer needed or are replaced by authorized use of completed permanent facilities.
Retain paragraph above or first paragraph and subparagraphs below.  Delete above if facilities are not available or their use is not permitted even if available.

C. Electric Power Service:  Provide electric power service and distribution system of sufficient size, capacity, and power characteristics required for construction operations.

3.2 SUPPORT FACILITIES INSTALLATION

A. Parking:  Provide temporary parking areas for construction personnel.

Revise first paragraph and subparagraphs below to satisfy soil-conservation district requirements.  See Evaluations.  Consult soil engineer about expected ground water.

B. Dewatering Facilities and Drains:  Comply with requirements of authorities having jurisdiction.  Maintain Project site, excavations, and construction free of water.

A. Dispose of rainwater in a lawful manner that will not result in flooding Project or adjoining properties nor endanger permanent Work or temporary facilities.

B. Remove snow and ice as required to minimize accumulations.

C. Project Identification and Temporary Signs:  Provide Project identification and other signs.  Install signs where indicated to inform public and individuals seeking entrance to Project.  Unauthorized signs are not permitted.

If required, insert a list of necessary signs and add Project-specific provisions such as special graphics and special lighting.  See Evaluations.

A. Provide temporary, directional signs for construction personnel and visitors.

B. Maintain and touchup signs so they are legible at all times.

Retain paragraph above or below.  See Evaluations.

D. Waste Disposal Facilities:  Provide waste-collection containers in sizes adequate to handle waste from construction operations.  Comply with requirements of authorities having jurisdiction.

Delete paragraph and subparagraph below for low-rise construction of less than three stories.  Subparagraph could be important for cost-plus contracts.

E. Lifts and Hoists:  Provide facilities necessary for hoisting materials and personnel.

A. Truck cranes and similar devices used for hoisting materials are considered "tools and equipment" and not temporary facilities.

Paragraph below is simplest method of specifying temporary elevator use.

F. Temporary Stairs:  Until permanent stairs are available, provide temporary stairs where ladders are not adequate.

Individual Project circumstances may require use of other construction aids and miscellaneous facilities, such as scaffolds, platforms, swing stages, ramps and bridges, incidental sheeting and shoring, and demolition waste chutes.  Add requirements to suit Project.  See Evaluations.

3.3 SECURITY AND PROTECTION FACILITIES INSTALLATION

A. Environmental Protection:  Provide protection, operate temporary facilities, and conduct construction in ways and by methods that comply with environmental regulations and that minimize possible air, waterway, and subsoil contamination or pollution or other undesirable effects.

A. Comply with work restrictions specified in Division 1 Section "Summary."

B. Temporary Erosion and Sedimentation Control:  Comply with requirements specified in Division 2 Section "Site Clearing."

Retain paragraph above or paragraph and subparagraph below.

C. Temporary Erosion and Sedimentation Control:  Provide measures to prevent soil erosion and discharge of soil-bearing water runoff and airborne dust to adjacent properties and walkways, according to requirements of authorities having jurisdiction.

A. Inspect, repair, and maintain erosion- and sedimentation-control measures during construction until permanent vegetation has been established.

Insert local regulations for environmental protection.  Restrictions on noisemaking activities should be specified in Division 1 Section "Summary."

D. Stormwater Control:  Comply with authorities having jurisdiction.  Provide barriers in and around excavations and subgrade construction to prevent flooding by runoff of stormwater from heavy rains.

Retain paragraph above or below.

Delete paragraph below only if cost considerations are paramount.  Revise if specific pests, such as termites or pigeons, are known to be a problem.

E. Pest Control:  Engage pest-control service to recommend practices to minimize attraction and harboring of rodents, roaches, and other pests and to perform extermination and control procedures at regular intervals so Project will be free of pests and their residues at Substantial Completion.  Obtain extended warranty for Owner.  Perform control operations lawfully, using environmentally safe materials.

Delete paragraph and subparagraphs below if not required.

F. Site Enclosure Fence:  Before construction operations begin, furnish and install site enclosure fence in a manner that will prevent people and animals from easily entering site except by entrance gates.

A. Extent of Fence:  As required to enclose entire Project site or portion determined sufficient to accommodate construction operations.

Option in subparagraph below is only for projects connected to existing construction.

B. Maintain security by limiting number of keys and restricting distribution to authorized personnel.  Provide Owner with one set of keys.

Retain first paragraph below because enclosure fence does not always provide adequate security against theft and vandalism.

G. Security Enclosure and Lockup:  Install substantial temporary enclosure around partially completed areas of construction.  Provide lockable entrances to prevent unauthorized entrance, vandalism, theft, and similar violations of security.

H. Barricades, Warning Signs, and Lights:  Comply with requirements of authorities having jurisdiction for erecting structurally adequate barricades, including warning signs and lighting.

I. Temporary Enclosures:  Provide temporary enclosures for protection of construction, in progress and completed, from exposure, foul weather, other construction operations, and similar activities.  Provide temporary weathertight enclosure for building exterior.

A. Where heating or cooling is needed and permanent enclosure is not complete, insulate temporary enclosures.

Paragraph and subparagraphs below specify currently accepted requirements; revise to suit local conditions.  See Evaluations.

J. Temporary Fire Protection:  Install and maintain temporary fire-protection facilities of types needed to protect against reasonably predictable and controllable fire losses.  Comply with NFPA 241.

A. Prohibit smoking in hazardous fire-exposure areas.

B. Supervise welding operations, combustion-type temporary heating units, and similar sources of fire ignition according to requirements of authorities having jurisdiction.

C. Develop and supervise an overall fire-prevention and -protection program for personnel at Project site.  Review needs with local fire department and establish procedures to be followed.  Instruct personnel in methods and procedures.  Post warnings and information.

D. Provide temporary standpipes and hoses for fire protection.  Hang hoses with a warning sign stating that hoses are for fire-protection purposes only and are not to be removed.  Match hose size with outlet size and equip with suitable nozzles.

If fire-suppression sprinkler systems or other permanent fire-protection systems are used, insert specific requirements.

3.4 OPERATION, TERMINATION, AND REMOVAL

First paragraph below is important if allowances for metered use of temporary facilities have been established.

A. Supervision:  Enforce strict discipline in use of temporary facilities.  To minimize waste and abuse, limit availability of temporary facilities to essential and intended uses.

B. Maintenance:  Maintain facilities in good operating condition until removal.

Delete first paragraph below if not required.

C. Operate Project-identification-sign lighting daily from dusk until 12:00 midnight.

D. Temporary Facility Changeover:  Do not change over from using temporary security and protection facilities to permanent facilities until Substantial Completion.

E. Termination and Removal:  Remove each temporary facility when need for its service has ended, when it has been replaced by authorized use of a permanent facility, or no later than Substantial Completion.  Complete or, if necessary, restore permanent construction that may have been delayed because of interference with temporary facility.  Repair damaged Work, clean exposed surfaces, and replace construction that cannot be satisfactorily repaired.

A. Materials and facilities that constitute temporary facilities are property of Contractor.  Owner reserves right to take possession of Project identification signs.

Consider inserting specific removal requirements, as illustrated in first subparagraph below.

B. At Substantial Completion, clean and renovate permanent facilities used during construction period.  Comply with final cleaning requirements specified in Division 1 Section "Closeout Procedures."

Review final draft of this Section with Owner.  Advise Owner that any exceptions to its provisions might translate into costs borne by Owner.

END OF DOCUMENT 015000
TEMPORARY FACILITIES AND CONTROLS

015000 - 1

