Century Park South

University of Southern Mississippi

Central Plant

Network Closet:

1 each Master Control Panel Persona V1000

1 each Power Supplies Altronix AL600ULPD16

Door: CP008b

1 each Sargent by Persona Exit Device PG8877 F ETL 626 RHR

1 each DPS Securitron Door Position Switch

1 each Motion Sensor BEA R2E-100

( Door position Switch and Motion Sensor to be connected to the V200)

Door: CP008a

1 each Sargent by Persona Exit Device PG8877 F ETL 626 RHR

1 each DPS Securitron Door Position Switch

1 each Motion Sensor BEA R2E-100

( Door position Switch and Motion Sensor to be connected to the V200)

Door: CP002, CP004a, CP004b, CP006, CP007

5 each Mortise Locks Persona VCP4055 x 626

Century Park South

University of Southern Mississippi

Police Kiosk

Doors: PK001, PK002

2 each Mortise Locks Persona VCP4055 x 626

Century Park South

University of Southern Mississippi

Building “A”

Health Care Clinic

Door: HC100b 		Changing to 1 single door

1 each Readers Persona W644

1 each Door Controller Persona V100

1 each Reader Power Supply PS 5R AO5

1 each DPS Securitron Door Position Switch

Door: HC162a

1 each Readers Persona W644

1 each Door Controller Persona V100

1 each Reader Power Supply PS 5R AO5

1 each DPS Securitron Door Position Switch

Door: HC100a 	Changing to 1 single door

1 each DPS Securitron Door Position Switch

1 each Monitor Controller Persona V200

Door: HC100c

1 each Reader Persona W644

1 each Reader Power Supply PS 5R AO5

1 each DPS Securitron Door Position Switch

(Use V100 from Door HC162a)

Door: HC160c

1 each Reader Persona W644

1 each Door Controller Persona V100

1 each Reader Power Supply PS 5R AO5

1 each DPS Securitron Door Position Switch

Door: HC117

1 each Mortise Locks Persona VCP4055 x 626

Door: HC100d

1 each Reader Persona W644

1 each Reader Power Supply PS 5R AO5

1 each DPS Securitron Door Position Switch

(Use V100 from Door HC160c)

Century Park South

University of Southern Mississippi

Building “A”

Health Care Clinic continued

Door: HC119, HC109, HC110, HC101, HC102, HC104, HC106, HC111a, HC111b, HC114, HC115, HC120, HC125, HC131, HC139, HC144, HC147, HC156B, HC156C

19 each Mortise Locks Persona VCP4055 x 626

Door: HC160a, HC161a, HC161b, HC162b

According to spec’s there is to be a release button at receptionist desk.

4 each Reader Persona W644

2 each Door Controller Persona V100

2 each Reader Power Supply PS 5R AO5

Room: HC126 Shot/Triage Medicine Cabinet

2 each Aperio Cabinet Locks HES K100-621

1 each Door Controller Persona V100

200 each Prox/Mag Swipe Cards (For medical Personnel)

Telephone Video to Reception Desk

Door: HC162a

1 each Aiphone JMS-4AEDF

NOTE: Base Station to be mounted at Nurse Reception Desk.

Century Park South

University of Southern Mississippi

Building “A”

Access Control Data Room

1 each Master Control Panel Persona V1000

6 each Power Supplies Altronix AL600ULPD16

1 each UPS Tripp Lite Smart 1500RMXL2UA

First Floor

Door: A100Aa

1 each Reader Persona W644

1 each Door Controller V100

1 each Reader Power Supply PS 5R AO5

1 each DPS Securitron Door Position Switch

1 each Motion Sensor BEA R2E-100

Door: A100Ab

1 each DPS Securitron Door Position Switch

(Use V200 from HC100a)

Door: A124b Single Door MD x MF 3’8” x 8’6” RHR

1 each Sargent by Persona Exit Device PG8877 G (3’8” Door) ETL 626 RHR

Door: A101b, A119a

2 each Mortise Locks Persona VCP4055 x 626

Door: A115b, A120b, A171

3 each DPS Door Position Switch

(Use V200 from HC100a)

Door: A104b     Read in / Read out

1 each Mortise Cylinder 1 1/8” LSA USM Keyway

1 each Electric Strike HES 1006 x 626

2 each Reader Persona W644

1 each Door Controller Persona V100

1 each Reader Power Supply PS 5R AO5

Century Park South

University of Southern Mississippi

Building “A”

First Floor continued

Door: A120a

1 each Rim Cylinder LSA USM Keyway

1 each Reader Persona W644

1 each DPS Securitron Door Position Switch

(Use V100 from Door A104b)

(Use P5 5R AO5 from Door A104b)

Door: A100e

1 each Rim Cylinder LSA USM Keyway

1 each Reader Persona W644

1 each Door Controller Persona V100

1 each Reader Power Supply PS 5R AO5

1 each DPS Securitron Door Position Switch

Door: A115a    Deleting this door heading completely

1 each Rim Cylinder LSA USM Keyway

1 each Reader Persona W644

1 each DPS Securitron Door Position Switch

1 each Motion Sensor BEA R2E-100

(Use V100 from Door A100e)

(Use PS 5R AO5 from Door A100e)

Door: A102a, A104a, A125, A126, A127, A129, A131, A128

8 each Mortise Locks Persona VCP4055 x 626

Door: A124a

1 each Mortise Locks Persona VCP4055 x 626

Doo: A108

1 each Mortise Cylinder LSA USM Keyway

Door: A106

1 each Electric Strike HES 1006

Door: A106a, A130a, A130b, A102b

4 each Mortise Locks Persona VCP4055 x 626

Century Park South

University of Southern Mississippi

Building “A”

First Floor continued

Door A113A

1 each Electric Strike HES 1006

1 each Reader Persona W644

1 each Door Controller Persona V100

1 each Reader Power Supply PS 5R AO5

1 each Motion Sensor BEA R2E-100

Door: A101, A105, A107, A109, A110, A111, A112, A114, A116, A118

10 each Mortise Locks Persona VCP4055 x 626

Second Floor

Doors: A215, A220, A271

3 each Mortise Cylinder 1 1/8’ LSA USM Keyway

3 each Reader Persona W644

2 each Door Controller Persona V100

2 each Reader Power Supply PS 5R AO5

3 each DPS Securitron Door Position Switch

Door: A200b, A200d

2 each Rim Cylinder LSA USM Keyway

2 each Reader Persona W644

1 each Door Controllers Persona V100

1 each Reader Power Supply PS 5R AO5

2 each DPS Securitron Door Position Switch

Door: A222, A242, A252, A224, A226, A228, A244, A246

8 each Mortise Locks VCP4055 x 626

Door: A213A, A202A (add this door)

1 each Electric Strike HES 1006

1 each Reader Persona W644

(Use V100 from Door A271)

(Use PS 5R AO5 from Door A271)

Century Park South

University of Southern Mississippi

Building “A”

Second Floor continued

Door: A204, A205, A206, A207, A208, A209, A210, A211, A212, A213 (Remove), A214, A216, A218, A219, A221, A223, A225, A227, A229, A230, A231, A232, A233, A234, A235, A236, A237, A238, A239, A241, A243, A245, A247, A249, A251, A253, A255, A257, A259, A260, A261, A262, A263, A264 (add this door), A265, A266, A267, A268, A269 A270, A240

50 each Mortise Locks Persona VCP 4055 x 626

Third Floor

Fourth Floor

Door: A315, A320, A371, A415, A420, A471

6 each Rim Cylinders LSA USM Keyway

6 each Readers Persona W644

3 each Door Controller Persona V100

3 each Reader Power Supply PS 5R AO5

6 each DPS Securitron Door Position Switch

Door: A300b, A300d, A400b, A400d

4 each Rim Cylinders LSA USM Keyway

4 each Readers Persona W644

2 each Door Controller Persona V100

2 each Reader Power Supply PS 5R AO5

4 each DPS Securitron Door Position Switch

Door: A328, A342, A324, A344, A346, A326, A428, A442, A424, A444, A446, A426

12 each Mortise Locks Persona VCP 4055 x 626

Door: A301,A302 (add this door), A303, A304, A305, A306, A307, A308, A309, A310, A311, A312, A313, A314, A316, A318, A319, A321, A323, A325, A327, A329a, A330, A331, A332, A333, A334, A335, A336, A337, A338, A339, A340, A341, A343, A345, A347, A349, A351, A353, A355, A356, A357, A358, A359, A360, A361, A362, A363, A364, A365, A366, A367, A368, A369, A370

A401, A402 (add this door), A403 A404, A405, A406, A407, A408, A409, A410, A411, A412, A413, A414, A416, A418, A419, A421, A423, A425, A427, A429a, A430, A431, A432, A433, A434, A435, A436, A437, A438, A439, A440, A441, A443, A445, A447, A449, A451, A453, A455, A456, A457, A458, A459, A460, A461, A462, A463, A464, A465, A466, A467, A468, A469, A470

112 each Mortise Locks Persona VCP 4055 x 626

Century Park South

University of Southern Mississippi

Building “A”

Fifth Floor

Door: A515, A520, A571

3 each Rim Cylinder LSA USM Keyway

3 each Readers Persona W644

2 each Door Controller Persona V100

2 each Reader Power Supply PS 5R AO5

3 each DPS Securitron Door Position Switch

Door: A500b, A500d

2 each Rim Cylinder LSA USM Keyway

2 each Readers Persona W644

1 each Door Controller Persona V100

1 each Reader Power Supply PS 5R AO5

2 each DPS Securitron Door Position Switch

Door: A528, A542, A524, A526, A544, A546

6 each Mortise Locks Persona VCP 4055 x 626

Door: A501, A502 (add this door), A503, A504, A505, A506, A507, A508, A509, A510, A511, A512, A513, A514, A516, A518, A519, A521, A523, A525, A527, A529a, A530, A531, A532, A533, A534, A535, A536, A537, A538, A539, A540, A541, A543, A545, A547, A549, A551, A553, A555, A556, A557, A558, A559, A560, A561, A562, A563, A564, A565, A566, A567, A568, A569, A570

56 each Mortise Locks Persona VCP 5044 x 626

Century Park South

University of Southern Mississippi

Building “B”

Lucky Day

Door: LD100a

1 each Readers Persona W644

1 each Door Controller V100

1 each Reader Power Supply PS 5R AO5

Door: LD100b

1 each DPS Securitron Door Position Switch

1 each Door Monitor Controller V200

Door: LD116 (remove ), LD118

1 each DPS Securitron Door Position Switch

(Use V200 from Door LD100b)

Door: LD115

1 each Reader Persona W644

(use V100 from door LD100a)

(use PS 5R AO5 from door LD100a)

Door: LD108, LD109, LD112b, LD112a, LD107, LD217a

6 each Mortise Locks Persona VCP 4055 x 626

Door: LD102, LD103, LD104, LD105

4 each Mortise Locks Persona VCP 4055 x 626

Door: LD217Ba, LD217c

2 each Mortise Locks Persona VCP 4055 x 626

Door: LD217D (Add this Door)

1 each Readers Persona W644

1 each Door Controllers Persona V100

1 each Reader Power Supply PS 5R AO5

1 each DPS Securitron Door Position Switch

1 each Motion Sensors BEA R2E-100

Century Park South

University of Southern Mississippi

Building “B”

Access Control Data Room

1 each Master Control Panel Persona V1000

6 each Power Supplies Altronix AL600ULPD16

1 each UPS Tripp Lite Smart 1500RMXL2UA

First Floor

Door: B100b

1 each Rim Cylinder LSA USM Keyway

1 each Reader Persona W644

1 each Door Controller Persona V100

1 each Reader Power Supply PS 5R AO5

1 each DPS Securitron Door Position Switch

Door: B100c

1 each Rim Cylinder LSA USM Keyway

1 each DPS Securitron Door Position Switch

(use V100 from Door B100b)

Door: B130b, B134b

2 each Mortise Locks Persona VCP 4055 x 626

Door: B115A, B135b, B151b, B154b

4 each Rim Cylinder LSA USM Keyway

4 each DPS Securitron Door Position Switch

1 each Door Monitor Controller Persona V200

Door: B119b, B101

2 each Mortise Locks Persona VCP 4055 x 626

Door: B102b

2 each Key-n-knob Cylinder LSA USM Keyway

1 each Electric Strike H.E.S. 1006

2 each Reader Persona W644

1 each Reader Power Supply PS 5R AO5

1 each Door Controller Persona V100

Century Park South

University of Southern Mississippi

Building “B”

Access Control Data Room

First Floor continued

Door: B135a, B151a, B154a

3 each Rim Cylinder LSA USM Keyway

3 each Reader Persona W644

2 each Door Controller Persona V100

2 each Reader Power Supply PS 5R AO5

3 each DPS Securitron Door Position Switch

Door: B100a

1 each Rim Cylinder LSA USM Keyway

1 each Reader Persona W644

1 each Reader Power Supply PS 5R AO5

1 each DPS Securitron Door Position Switch

(Use V100 from Door B154a)

Door: B102a, B119a, B103 (Add this door), B123, B125a, B125b, B125c, B130 (remove), B139, B141, B103, B104

11 each Mortise Locks Persona VCP 4055 x 626

Door: B128a

1 each Electric Strike H.E.S. 1006

Door: B105, B128b

2 each Mortise Locks Persona VCP 4055 x 626

Door: B130.A, B134A (add this door), B149.A,

3 each Electric Strike H.E.S. 1006

3 each Reader Persona W644

2 each Door Controller Persona V100

2 each Reader Power Supply PS 5R AO5

Door: B127, B129, B131, B133, B134a (remove), B140, B142, B143, B144, B145, B146, B147, B148, B149 (remove), B150

13 each Mortise Locks Persona VCP 4055 x 626

Century Park South

University of Southern Mississippi

Building “B”

Second Floor

Door: B215, B235, B251, B254, B200a, B200b

6 each Rim Cylinder LSA USM Keyway

6 each Reader Persona W644

3 each Door Controller Persona V100

3 each Reader Power Supply PS 5R AO5

6 each DPS Securitron Door Position Switch

Door: B221, B223, B241, B225, B239

5 each Mortise Locks Persona VCP4055 x 626

Door: B213, B249

2 each Electric Strikes H.E.S. 1006

2 each Readers Persona W644

1 each Door Controllers Persona V100

1 each Reader Power Supply PS 5R AO5

2 each DPS Securitron Door Position Switch

2 each Motion Sensors BEA R2E-100

Door: B201, B202, B203, B204, B205, B206, B207, B208, B209, B210, B211, B212, B217, B218, B219, B220, B222, B224, B226, B227, B228, B229, B230, B231, B232, B233, B234, B238, B240, B242, B243, B244, B245, B246, B247, B248, B250, B214

38 each Mortise Locks Persona VCP4055 x 626

Third Floor

Fourth Floor

Door: B300a, B315, B335, B351, B354, B400a, B415, B435, B451, B454

10 EACH Rim Cylinders LSA USM Keyway

10 each Readers Persona W644

5 each Door Controllers Persona V100

5 each Reader Power Supplies PS 5R AO5

10 each DPS Securitron Door Position Switch

10 each Motion Sensors BEA R2E-100

Century Park South

University of Southern Mississippi

Building “B”

Third Floor continued

Fourth Floor continued

Door: B300b, B400b

2 each Rim Cylinders LSA USM Keyway

2 each Readers Persona W644

1 each Door Controllers Persona V100

1 each Reader Power Supply PS 5R AO5

2 each DPS Securitron Door Position Switch

2 each Motion Sensors BEA R2E-100

Door: B321, B323, B339, B341, B325, B421, B423, B439, B441, B435 (remove)

9 each Mortise Locks Persona VCP4055 x 626

Door: B301, B302, B303, B304, B305, B306, B307, B308, B309, B310, B311, B312, B313, B314, B316, B317,

B318, B319, B320, B322, B324, B326, B327, B328, B329, B330, B331, B332, B333, B334, B338, B340, B342,

B343, B344, B345, B346, B347, B348, B349, B350

B401, B402, B403, B404, B405, B406, B407, B408, B409, B410, B411, B412, B413, B414, B416, B417, B418, B419, B420, B422, B424, B426, B427, B428, B429, B430, B431, B432, B433, B434, B438, B440, B442, B443, B444, B445, B446, B447, B448, B449, B450

82 each Mortise Locks Persona VCP 4055 x 626

Fifth Floor

Door: B515, B535, B551, B554

4 each Rim Cylinders LSA USM Keyway

4 each Reader Persona W644

2 each Door Controller Persona V100

2 each Reader Power Supply PS 5R AO5

4 each DPS Securitron Door Position Switch

Door: B500a, B500b

2 each Rim Cylinders LSA USM Keyway

2 each Reader Persona W644

1 each Door Controller Persona V100

2 each Reader Power Supply PS 5R AO5

Door: B521, B541, B525, B539, B523

5 each Mortise Locks Persona VCP 4055 x 626

Century Park South

University of Southern Mississippi

Building “B”

Fifth Floor continued

Door: B501, B502, B503, B504, B505, B506, B507, B508, B509, B510, B511, 5B12, B513, B514, B516, B517, B518, B519, B520, B522, B524, B526, B527, B528, B529, B530, B531, B532, B533, B534, B538, B540, B542, B543, B544, B545, B546, B547, B548, B549, B550

41 each Mortise Locks Persona VCP 4055 x 626

Century Park South

University of Southern Mississippi

Building “C”

Access Control Data Room

1 each Master Control Panel  V1000

6 each Power Supplies Altronix AL600ULPD16

1 each UPS Tripp Lite Smart1500 RMXL2UA

First Floor

Door: C101a

1 each Rim Cylinders LSA USM Keyway

1 each Reader Persona W644

1 each Door Controller Persona V100

1 each Reader Power Supply PS 5R AO5

1 each DPS Securitron Door Position Switch

Door: C101b

1 each DPS Securitron Door Position Switch

1 each Door Monitor Controller V200

Door: C100b

1 each Rim Cylinders LSA USM Keyway

1 each Reader Persona W644

1 each Reader Power Supply PS 5R AO5

1 each DPS Securitron Door Position Switch

(Use V100 from Door C101a)

Door: C122b

1 each Rim Cylinders LSA USM Keyway

1 each Sargent by Persona Exit Device PG8877 F ETL RHR 626

1 each DPS Securitron Door Position Switch (connect to V200 reference door C101b)

1 each Motion Sensor BEA R2E-100 (connect to V200 reference door C101b)

Door: C124b, C132b

2 each Mortise Locks Persona VCP 4055 x 626

Door: C134b

1 each DPS Securitron Door Position Switch

(Use V200 from C101b)

Century Park South

University of Southern Mississippi

Building “C”

First Floor continued

Door: C108b

1 each Electric Strike H.E.S. 1006

2 each Reader Persona W644

1 each Door Controller Persona V100

1 each Reader Power Supply PS 5R AO5

1 each DPS Securitron Door Position Switch

Door: C105, C134a

2 each Rim Cylinders LSA USM Keyway

2 each Reader Persona W644

1 each Door Controller Persona V100

1 each Reader Power Supply PS 5R AO5

2 each DPS Securitron Door Position Switch

Door: C100a

1 each Rim Cylinder LSA USM Keyway

1 each Reader Persona W644

1 each Door Controller Persona V100

1 each Reader Power Supply PS 5R AO5

1 each DPS Securitron Door Position Switch

Door: C108, C111, C115, C121, C122a, C124a, C130, C139, C106a, C109, C104

11 each Mortise Locks Persona VCP 4055 x 626

Door: C120

1 each Electric Strike H.E.S. 1006

Door: C125A, C131A, C132A

3 each Electric Strike H.E.S. 1006

3 each Reader Persona W644

2 each Door Controller Persona V100

2 each Reader Power Supply PS 5R AO5

3 each DPS Securitron Door Position Switch

3 each Motion Sensors BEA R2E-100

Door: C110, C112, C114, C116, C118, C127, C128, C129, C141

9 each Mortise Locks Persona VCP 4055 x 626

Century Park South

University of Southern Mississippi

Building “C”

Second Floor

Door: C202

1 each Mortise Locks Persona VCP 4055 x 626

Door: C 205, C234, C200a, C213

4 each Rim Cylinder LSA USM Keyway

4 each Reader Persona W644

2 each Door Controller Persona V100

2 each Reader Power Supply PS 5R AO5

4 each DPS Securitron Door Position Switch

Door: C203, C207, C211

3 each Mortise Locks Persona VCP 4055 x 626

Door: C220 (remove door)

1 each Electric Strike H.E.S. 1006

Door: C225, C231

2 each Electric Strikes H.E.S. 1006

2 each Reader Persona W644

1 each Door Controller Persona V100

1 each Reader Power Supply PS 5R AO5

Door: C206, C208, C210, C212, C214, C216, C218, C222, C224, C226, C227, C228, C229, C230, C232

15 each Mortise Locks Person VCP 4055 x 626

Third Floor

Fourth Floor

Door: C305, C334, C405, C434

4 each Rim Cylinder LSA USM Keyway

4 each Reader Persona W644

2 each Door Controller Persona V100

2 each Reader Power Supply PS 5R AO5

4 each DPS Securitron Door Position Switch

Century Park South

University of Southern Mississippi

Building “C”

Third and Fourth Floor continued

Door: C300a, C313, C400a, C413

4 each Rim Cylinder LSA USM Keyway

4 each Reader Persona W644

2 each Door Controller Persona V100

2 each Reader Power Supply PS 5R AO5

4 each DPS Securitron Door Position Switch

Door: C303, C307, C311, C403, C407, C411

6 each Mortise Locks Persona VCP 4055 x 626

Door: C320, C420 (remove doors)

2 each Electric Strikes H.E.S. 1006

Door: C302, C304, C306, C308, C310, C312, C314, C316, C318, C322, C324, C325, C326, C327, C328, C329, C330,

C331, C332, C341

C402, C404, C406, C408, C410, C412, C414, C416, C418, C422, C424, C425, C426, C427, C428, C429, C430, C431, C432, C441

40 each Mortise Locks Persona VCP 4055 x 626

Fifth Floor

Door: C505, C534, C500a, C513

4 each Rim Cylinder LSA USM Keyway

4 each Reader Persona W644

2 each Door Controller Persona V100

2 each Reader Power Supply PS 5R AO5

4 each DPS Securitron Door Position Switch

Door: C503, C507, C511

3 each Mortise Locks Persona VCP 4055 x 626

Door: C520 (remove door)

1 each Electric Strike H.E.S. 1006

Century Park South

University of Southern Mississippi

Building “C”

Fifth Floor continued

Door: C502, C504, C506, C508, C510, C512, C514, C516, C518, C522, C524, C525, C526, C527, C528, C529, C530,

C531, C532, C541

20 each Mortise Locks Persona VCP 4055 x 626

Century Park South

University of Southern Mississippi

Pedestrian Gates

2 each Read-in/ Read-out Gates:

2 each Securitron Maglock M62FGBFD

4 each Reader Persona W644

2 each Door Controller Persona V100

2 each Read Power Supply PS 5R AO5

3 each Pedestrian Gates:

3 each Securitron Maglocks M62FGBD

3 each Reader Persona W644

2 each Door Controller Persona V100

3 each Reader Power Supply PS 5R AO5

Locks and Reader are to be integrated with gate openers. Termination is to be coordinated with access control installer and gate opener installer.

Century Park South

University of Southern Mississippi

CCTV and Cameras

Building “A”

Access Control Room

2 each 		OV Solutions DVR OV 4U48 3TB 64G

1 each		OV Solutions DVR OV 4U32 3TB 64G

1 each 		OV Solutions Monitor DMON-22

First Floor

13 each 	Securecam Exit Sign Only

1 each 		Securecam Exit Sign SCI- MCC700 HPF EX-600 DWDR Double Lens

8 each		Securecam Exit Sign SCI- MCC700 HPF EX2-600 DWDR Single Lens

8 each 		Clinton Electronic Dome Cameras CE-D953

3 each		Securecam Motion Sign SCI- MCC700 HPF PIR-600 WDR Single Lens

2 each 	OV Solutions Elevator Corner Camera C-ACM 620

Second Floor

3 each 		Securecam Exit Sign Only

2 each 		Securecam Exit Sign SCI- MCC700 HPF EX-600 DWDR Double Lens

4 each		Securecam Exit Sign SCI- MCC700 HPF EX2-600 DWDR Single Lens

5 each		Securecam Motion Sign SCI- MCC700 HPF PIR-600 WDR Single Lens

2 each		Securecam Smoke Detector SCI- MCC700 HPF SD-600 DWDR Double Lens

Third Floor

3 each 		Securecam Exit Sign Only

2 each 		Securecam Exit Sign SCI- MCC700 HPF EX-600 DWDR Double Lens

4 each		Securecam Exit Sign SCI- MCC700 HPF EX2-600 DWDR Single Lens

5 each		Securecam Motion Sign SCI- MCC700 HPF PIR-600 WDR Single Lens

1 each		Securecam Smoke Detector SCI- MCC700 HPF SD-600 DWDR Double Lens

Fourth Floor

3 each 		Securecam Exit Sign Only

2 each 		Securecam Exit Sign SCI- MCC700 HPF EX-600 DWDR Double Lens

4 each		Securecam Exit Sign SCI- MCC700 HPF EX2-600 DWDR Single Lens

5 each		Securecam Motion Sign SCI- MCC700 HPF PIR-600 WDR Single Lens

1 each		Securecam Smoke Detector SCI- MCC700 HPF SD-600 DWDR Double Lens

Fifth Floor

3 each 		Securecam Exit Sign Only

2 each 		Securecam Exit Sign SCI- MCC700 HPF EX-600 DWDR Double Lens

4 each		Securecam Exit Sign SCI- MCC700 HPF EX2-600 DWDR Single Lens

5 each		Securecam Motion Sign SCI- MCC700 HPF PIR-600 WDR Single Lens

1 each		Securecam Smoke Detector SCI- MCC700 HPF SD-600 DWDR Double Lens

Century Park South

University of Southern Mississippi

CCTV and Cameras

Building “B”

Access Control Room

1 each 		OV Solutions DVR OV 4U48 3TB 64G

2 each		OV Solutions DVR OV 4U32 3TB 64G

1 each 		OV Solutions Monitor DMON-22

First Floor

9 each 		Securecam Exit Sign Only

1 each 		Securecam Exit Sign SCI- MCC700 HPF EX-600 DWDR Double Lens

4 each		Securecam Exit Sign SCI- MCC700 HPF EX2-600 DWDR Single Lens

3 each 		Clinton Electronic Dome Cameras CE-D953

7 each		Securecam Motion Sign SCI- MCC700 HPF PIR-600 WDR Single Lens

1 each 		Securecam Smoke Detector SCI- MCC700 HPF SD-600 DWDR Double Lens

3 each 	OV Solutions Elevator Corner Camera C-ACM 620

Second Floor

4 each 		Securecam Exit Sign Only

1 each 		Securecam Exit Sign SCI- MCC700 HPF EX-600 DWDR Double Lens

6 each		Securecam Exit Sign SCI- MCC700 HPF EX2-600 DWDR Single Lens

3 each		Securecam Motion Sign SCI- MCC700 HPF PIR-600 WDR Single Lens

1 each		Securecam Smoke Detector SCI- MCC700 HPF SD-600 DWDR Single Lens

Third Floor

3 each 		Securecam Exit Sign Only

1 each 		Securecam Exit Sign SCI- MCC700 HPF EX-600 DWDR Double Lens

6 each		Securecam Exit Sign SCI- MCC700 HPF EX2-600 DWDR Single Lens

3 each		Securecam Motion Sign SCI- MCC700 HPF PIR-600 WDR Single Lens

1 each		Securecam Smoke Detector SCI- MCC700 HPF SD-600 DWDR Single Lens

Fourth Floor

3 each 		Securecam Exit Sign Only

1 each 		Securecam Exit Sign SCI- MCC700 HPF EX-600 DWDR Double Lens

6 each		Securecam Exit Sign SCI- MCC700 HPF EX2-600 DWDR Single Lens

3 each		Securecam Motion Sign SCI- MCC700 HPF PIR-600 WDR Single Lens

1 each		Securecam Smoke Detector SCI- MCC700 HPF SD-600 DWDR Single Lens

Fifth Floor

3 each 		Securecam Exit Sign Only

1 each 		Securecam Exit Sign SCI- MCC700 HPF EX-600 DWDR Double Lens

6 each		Securecam Exit Sign SCI- MCC700 HPF EX2-600 DWDR Single Lens

3 each		Securecam Motion Sign SCI- MCC700 HPF PIR-600 WDR Single Lens

1 each		Securecam Smoke Detector SCI- MCC700 HPF SD-600 DWDR Single Lens

Century Park South

University of Southern Mississippi

CCTV and Cameras

Building “C”

Access Control Room

1 each 		OV Solutions DVR OV 4U48 3TB 64G

1 each		OV Solutions DVR OV 4U32 3TB 64G

1 each 		OV Solutions Monitor DMON-22

First Floor

7 each 		Securecam Exit Sign Only

1 each 		Securecam Exit Sign SCI- MCC700 HPF EX-600 DWDR Double Lens

5 each		Securecam Exit Sign SCI- MCC700 HPF EX2-600 DWDR Single Lens

4 each		Securecam Motion Sign SCI- MCC700 HPF PIR-600 WDR Single Lens

2 each 		Securecam Smoke Detector SCI- MCC700 HPF SD-600 DWDR Double Lens

2 each 		OV Solutions Elevator Corner Camera C-ACM 620

Second Floor

4 each 		Securecam Exit Sign Only

2 each 		Securecam Exit Sign SCI- MCC700 HPF EX-600 DWDR Double Lens

5 each		Securecam Exit Sign SCI- MCC700 HPF EX2-600 DWDR Single Lens

6 each		Securecam Motion Sign SCI- MCC700 HPF PIR-600 WDR Single Lens

2 each		Securecam Smoke Detector SCI- MCC700 HPF SD-600 DWDR Single Lens

Third Floor

2 each 		Securecam Exit Sign Only

2 each 		Securecam Exit Sign SCI- MCC700 HPF EX-600 DWDR Double Lens

5 each		Securecam Exit Sign SCI- MCC700 HPF EX2-600 DWDR Single Lens

3 each		Securecam Motion Sign SCI- MCC700 HPF PIR-600 WDR Single Lens

1 each		Securecam Smoke Detector SCI- MCC700 HPF SD-600 DWDR Single Lens

Fourth Floor

2 each 		Securecam Exit Sign Only

2 each 		Securecam Exit Sign SCI- MCC700 HPF EX-600 DWDR Double Lens

5 each		Securecam Exit Sign SCI- MCC700 HPF EX2-600 DWDR Single Lens

3 each		Securecam Motion Sign SCI- MCC700 HPF PIR-600 WDR Single Lens

1 each		Securecam Smoke Detector SCI- MCC700 HPF SD-600 DWDR Single Lens

Fifth Floor

2 each 		Securecam Exit Sign Only

2 each 		Securecam Exit Sign SCI- MCC700 HPF EX-600 DWDR Double Lens

5 each		Securecam Exit Sign SCI- MCC700 HPF EX2-600 DWDR Single Lens

3 each		Securecam Motion Sign SCI- MCC700 HPF PIR-600 WDR Single Lens

1 each		Securecam Smoke Detector SCI- MCC700 HPF SD-600 DWDR Single Lens

Century Park South

University of Southern Mississippi

Paging System

Building “A”

Front Desk

1 each	MBS1000A Desktop Microphone

1 each 	GS250 Amplifier

First Floor

33 each	Speakers S86T725PG8WVK

Second Floor

21 each 	Speakers S86T725PG8WVK

Third Floor

21 each	Speakers S86T725PG8WVK

Fourth Floor

21 each	Speakers S86T725PG8WVK

Fifth Floor

21 each	Speakers S86T725PG8WVK

Century Park South

University of Southern Mississippi

Paging System

Building “B”

Front Desk

1 each	MBS1000A Desktop Microphone

1 each 	GS250 Amplifier

First Floor

11 each	Speakers S86T725PG8WVK

Second Floor

14 each 	Speakers S86T725PG8WVK

Third Floor

14 each	Speakers S86T725PG8WVK

Fourth Floor

14 each	Speakers S86T725PG8WVK

Fifth Floor

14 each	Speakers S86T725PG8WVK

Century Park South

University of Southern Mississippi

Paging System

Building “C”

Front Desk

1 each	MBS1000A Desktop Microphone

1 each 	GS250 Amplifier

First Floor

11 each	Speakers S86T725PG8WVK

Second Floor

11 each 	Speakers S86T725PG8WVK

Third Floor

11 each	Speakers S86T725PG8WVK

Fourth Floor

11 each	Speakers S86T725PG8WVK

Fifth Floor

11 each	Speakers S86T725PG8WVK