554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

July 31, 2025

Jeff & Pam Puckett

Latter & Blum

Ref: 400 Longleaf

Slidell, La. 70458

Construction:

One-story, wood frame, brick veneer, vinyl/carpet flooring, with a composition shingle roof on a concrete foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to the brick veneer showing signs of cracks.

Findings:

Upon inspection, on the right side of the homes brick veneer/mortar was noted to have cracks below the bathroom window down into the foundation. The brick veneer/mortar on the left side of the home below the kitchen window and kitchen sink window was noted to have cracks that run down to the foundation.

The gutters and downspouts on the rear of the home drain directly onto the ground and water is getting under the foundation causing washout and expansion and contraction of the ground.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

Recommendation:

It is our opinion that the structural integrity of this home is currently sound, but the foundation will expand and contract with the ground movement and any standing water.  It is recommended when there is more than a 3” elevation difference (tripping hazard) to have the home stabilized with screw pilings. The home is level.

It is recommended to address the items below.

Items to be addressed are as follows:

- Seal all the cracks in the brick veneer and foundation with a non-shrinking grout, to prevent insects from entering the home.

- Add gutters and down spouts around the whole home with subsurface drainage to the street.

- Remove any trees and roots within 10’of the foundation.

See attached pictures

Sincerely,

Brian Mistich, P.E.