ST. TAMMANY FIRE PROTECTION DISTRICT NO. 1

NEW TRAINING FACILITY

34780 S. RANGE ROAD 

SLIDELL LA  70460

Copyright 2010 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 123000
CASEWORK
Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Section includes kitchen cabinets.

1.2 DEFINITIONS

Retain terms that remain after this Section has been edited for a project.

A. MDF: Medium-density fiberboard.

1.3 ACTION SUBMITTALS

A. Product Data: For the following:

Delete items for which Product Data are not required.

1. Cabinets.

2. Cabinet hardware.

1.4 INFORMATIONAL SUBMITTALS

A. Product Certificates: For casework.

PART 2 -  PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products. For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

2.1 CABINETS

Copy this article and re-edit it to specify different cabinet types if using more than one type for same Project.

If using a semiproprietary or proprietary specification, it is unnecessary to include descriptive requirements in other Part 2 articles that merely restate manufacturer's standard features. If using a nonproprietary specification, especially for government work, descriptive requirements in this and other Part 2 articles must be included to specify the salient characteristics used to determine whether products proposed for use by Contractor are equivalent.

Retain "Products" Paragraph and list of manufacturers and products below to require specific products or a comparable product from other manufacturers.

A. Basis of Design: Subject to compliance with requirements, provide product indicated on Drawings or comparable product by one of the following:

· Cabinetry by Diamond
· Aristokraft Cabinetry

· Singer Kitchens

B.
Product Line:  Prelude


Door Style: Wallace
2.2 CABINET MATERIALS

A. General:

Retain "Adhesives and Composite Wood and Agrifiber Products" Subparagraph below if required for LEED-NC, LEED-CI, or LEED-CS Credit IEQ 4.4.

1. Adhesives and Composite Wood and Agrifiber Products: Do not use products that contain urea formaldehyde.

Retain first two subparagraphs below if required for LEED for Schools Credit IEQ 4.1 and Credit IEQ 4.4.

2. Hardwood Lumber: Kiln dried to 7 percent moisture content.

3. Softwood Lumber: Kiln dried to 10 percent moisture content.

4. Hardwood Plywood: HPVA HP-1; made with adhesive containing no urea formaldehyde.

5. Particleboard: ANSI A208.1, Grade M-2; made with binder containing no urea formaldehyde.

6. MDF: ANSI A208.2, Grade MD; made with binder containing no urea formaldehyde.

B. Exposed Materials:

1. Exposed Wood Species: Maple
a. Select materials for compatible color and grain. Do not use two adjacent exposed surfaces that are noticeably dissimilar in color, grain, figure, or natural character markings.

b. Staining and Finish: As selected by Architect from manufacturer's full range.

2. Solid Wood: Clear hardwood lumber of species indicated, free of defects.

3. Plywood: Hardwood plywood with face veneer of species indicated, with Grade A faces and Grade C backs of same species as faces.

4. Plastic Laminate: Particleboard faced with high-pressure decorative laminate complying with NEMA LD 3.

a. Colors, Textures, and Patterns: As selected by Architect from cabinet manufacturer's full range.

C. Concealed Materials: Solid wood or plywood, of any hardwood or softwood species, with no defects affecting strength or utility; particleboard; MDF; or hardboard.

2.3 CABINET HARDWARE

Revise this article to specify hardware in more detail if required. Consider deleting this article if cabinets are specified by manufacturer's name or model designation that includes hardware selection.

A. General: Manufacturer's standard units complying with BHMA A156.9, of type, size, style, material, and finish as indicated by manufacturer's designations.

B. Pulls: Back-mounted decorative pulls.  Basis of design:  Diamond Pull H63.
C. Hinges: Concealed European-style, self-closing hinges.

D. Drawer Guides: Epoxy-coated-metal, self-closing drawer guides; designed to prevent rebound when drawers are closed; with nylon-tired, ball-bearing rollers; and complying with BHMA A156.9, Type B05011 or Type B05091.

Insert paragraphs describing other hardware and accessories if required.

PART 3 -  EXECUTION

3.1 INSTALLATION

A. Install cabinets with no variations in flushness of adjoining surfaces; use concealed shims. Where cabinets abut other finished work, scribe and cut for accurate fit. Provide filler strips, scribe strips, and moldings in finish to match cabinet face.

B. Install cabinets without distortion so doors and drawers fit the openings, are aligned, and are uniformly spaced. Complete installation of hardware and accessories as indicated.

C. Install cabinets level and plumb to a tolerance of 1/8 inch in 8 feet.

D. Fasten cabinets to adjacent units and to backing.

Revise requirements in subparagraph below as necessary for seismic restraint of cabinets. Delete below if hanging cleats are used and are detailed on Drawings.

1. Fasten wall cabinets through back, near top and bottom, and at ends not more than 16 inches o.c. with No. 10 wafer-head screws sized for not less than 1-1/2-inch  penetration into wood framing, blocking, or hanging strips.

3.2 ADJUSTING AND CLEANING

A. Adjust cabinets and hardware so doors and drawers are centered in openings and operate smoothly without warp or bind. Lubricate operating hardware as recommended by manufacturer.

END OF SECTION 123000
	CASEWORK
	123000 - 1



