ST. TAMMANY FIRE PROTECTION DISTRICT NO. 1
NEW TRAINING FACILITY
34780 S. RANGE ROAD 

SLIDELL LA  70460
[image: image1]

SECTION 015813
PROJECT IDENTIFICATION SIGN
PART 1 -  GENERAL

1.1 SUMMARY

A. Section Includes:  Project Identification Sign
PART 2 -  PRODUCTS
2.1 SIGN MATERIAL

A. Graphic Design, Content, Colors, Lettering Style:  See attached sketch.

B. Sign Size:  nominal 4 feet by 8 feet.

C. Quantity:  Provide 2 Project identification signs.

D. Support Structure, Framing, and Trim: All pressure treated, sizes as specified on sign detail at end of this Section.

E. Paint and Primers:  Exterior quality, gloss alkyd enamel, two coats, paints shall be approved by Architect.

1. One coat primer:  Exterior Alkyd Wood Primer:

a. Sherwin Williams: Exterior Oil Wood Primer, A-100.

2. Two topcoats:  Exterior Alkyd Enamel (Gloss): (Gloss Level 6).

a. Sherwin Williams; Seaguard 1000 Marine Enamel, Industrial & Marine.

b. Color shall be Sherwin Williams SW 6524 “Commodor” or approved equal.

F. Concrete:  Concrete for sign post footings shall be minimum 3,000 psi.  Comply with ASTM C 94; ACI 301, “Specification for Structural Concrete”; ACI 117, “Specifications for Tolerances for Concrete Construction and Materials”; and CRSI’s “Manual of Standard Practice.”

G. Fasteners: Fasten sign with stainless steel screws and stainless steel cup washers.  All other fasteners and accessories shall be hot dip galvanized.

PART 3 -  EXECUTION
3.1 INSTALLATION

A. Install Project identification signs within 30 days after date fixed by Notice to Proceed.

B. Erect Project identification signs in locations as directed by Architect.

C. Install signs plumb and level.  Anchor securely.

D. Paint all exposed surfaces of sign, supports, and framing according to manufacturer’s and MPI’s recommendations.

3.2 MAINTENANCE

A. Maintain signs and supports and keep clean.  Repair damage.  Replace deteriorated signs during course of project as directed by Architect.

3.3 REMOVAL

A. Remove signs, framing, supports and foundation at completion of Project and restore the area.


[image: image4.png]100x100MM (4%
FRESSRRETED
FOST: COLOR. Bl

MITRE CORNER.

75MM (3)
[EERRE

TN,
Gyt

75MM (3)

g
Nore

sore: i
PAIT ALL OTHER WOOD 1
‘SURFACES WITH ONE COAT iCITY AND STATE |
EXT. PRIMER AND TWO COATS 1
GLOSS WHITE ENAMEL

NOTE:

AL LETTERING SHALL BE

EVENLY SPAGED.
NOTE

LOCATIONS.

73MM (3
[ETERAE

T220mm

o]

ENAME OF FACILITY
| SECOND LINE

OCATION OF FACILITY:

|
|
|
[CITY AND STATE

ST TAVIMANY FIRE
BROTECTION DiSTRICT

PAINTED FIELD:
ol

PANTED 100w (3 /)
STRIPE: COLOR, BLUF

| 1comm 4 noveces

PAINTED FIELD:
‘COLOR, WriTE

PANTED LETTERS:
Sz, STws B,
COLOR, BUE: TYFICAL
FOR ALL LETTERING

4 THR, EXT. PLYWOOD.

Zi0mm

Zadomm

GROUND LEVEL

100x100me (4]
| — reessure ke
FOST: COLOR, WHITE'

e

— A

PROJECT IDENTIFICATION SIGNBOARD

[

WITHOUT RENDERING
NT.S.

Si5mm






[image: image2]
END OF DOCUMENT 015813
[image: image3.png]P

%4 BB o

| 1000m iy
AR rosr

|
|
|
xote: Ll 1000 00mn e
POSTS AND BRACES SHAL B2 1 FOST BEYOND.
i
i
it

PRESSURE TREATED, ALL
FASTENINGS SHALL BE ZINC. | iomgmwoo
COATED. PANEL SIGNBOARD
INCLUDE OPTIONAL BRACING IN
UNSTABLE SOIL OR HIGH WIND
ENVIRONMENTS,

5041 00mm (2%4) BRACE

_— 50¢100mm (2447 BRACE

19mm () PLAWOOD

L\oouommwm

PRESSURE TREATED FOST

(AN _TOP VIEW
e/

20mm (%) 8OLTS.

i1
.
.
o
o
.
PROJECT P
IDENTIFICATION. i 500100mm (2367
SIGNBOARD SUPPORT L BB
DETAILS
NS (1 _SIDE VIEW




PROJECT IDENTIFICATION SIGN

015813 - 1

