Twin Oaks

Bldg. #8

1) Fire-caulk all alarm and cable wire penetrations through wood. Some electrical wiring and piping also need to be checked for total sealant. 

2) Fill all tub boxes under tubs.

3) Fire dampers need to be installed in all vents for the bathrooms.

4) Install straps on vent & sewer stacks and secure hot and cold water lines.

5) All third floor can lights in kitchen must be in a rated light fixture.

6) Kill mold found on all ceiling joist, stud walls and floors.

7) Install all hat channels on ceilings for sheet rock.

8) Install all water heaters.

9) Install insulation in all walls and ceilings. 

Bldg. #11

1) Fire-caulk all alarm and cable wire penetrations through wood. Some electrical wiring and piping also need to be checked for total sealant. 

2) Fill all tub boxes under tubs.

3) Fire dampers need to be installed in all vents for the bathrooms.

4) Install straps on vent & sewer stacks and secure hot and cold water lines.

5) All third floor can lights in kitchen must be in a rated light fixture.

6) Kill mold found on all ceiling joist, stud walls and floors.

7) Install all hat channels on ceilings for sheet rock.

8) Install all water heaters.

9)    Install insulation in all walls and ceilings.

