554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

To: 	The Restore Louisiana Homeowner Assistance Program

For:      Twenty 9 Eleven Contracting Service

1000 Caruso Blvd.

Slidell, La. 70458

Ref:     Arlene Wilkerson

500 Ave F

Kentwood, La. 70444

Construction:

3,131 Sq. Ft. One-story, Wood frame, Wood veneer, Wood flooring, with a composition shingle roof on a pier and beam foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to it being currently renovated and the floors not being level.

Findings:

An overall visual inspection of the exterior of the home was conducted and noted that some of the piers supporting the beams have been previously shimmed and some have settled. The interior floors were noted to have some settling up to 4-1/2”.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

Recommendation:

It is our opinion that the structural integrity of this home is currently sound, but the foundation will expand and contract with the ground movement and any standing water.

Items to be addressed are as follows:

- Level all piers that have settled.

- Add minimum amount of fill under the home to provide proper drainage of rainwater away from the pier and beam foundation and keep dry.

- Add gutters and down spouts to direct the water away from the home and out to the street.

See attachments;

Pictures

Sincerely,

Brian Mistich, P.E.