Many of us have a Garmin 430 in our Lancairs. I had a message displayed on
my Garmin 430 "memory battery low". That message indicates the internal battery
needs to be replaced. If the internal battery is not replaced, the 430 can lose
com and gps functions even in the presence of good airplane battery power. 
Garmin has a flat rate service charge of $800 for the GNS430. I chose to
replace the battery myself.

The battery is a Sanyo CR14250SE-FT. It is a 3 volt 850mAH lithium computer battery. It has 1 negative pin and 2 positive pins.

The process involved removal of the 430 from the tray, opening the 430 by
removal of about a dozen screws, and disconnecting a couple of computer ribbon
cables to allow removal of the board securing the battery. Once the board is
removed, the battery may be removed from the board by heating the soddered
battery pins at the board. The new battery must be soddered back to the board. 
Both positive battery pins must make good contact in addition to the negative. 
If only one positive pin makes good contact, you will get an error message of
either "memory battery low" or "com not responding". This entire process takes
only around 15 minutes once familiar.