Product:  30022 T6X - E-P Express control model as manufactured by Pellerin Milnor Corp.

Maximum Capacity - lb. (kg): 60 (27).

Cylinder Diameter x Depth - inches (mm): 30 x 22 (762 x 559).

Gross Cylinder volume - cu. ft. (L): 9 (255).

Motor - HP (kW): 4 (2.98).

Wash Speed - RPM: 38.

Distribution Speed - RPM: 65.

Extract - RPM: 573.

Extract G Forces: 140.

Inlet Valve - inch (mm): 0.75 (19).

Drain Valve - inches (mm): 3 (76).

Size:  Overall Width - inches (mm): 34.5 (876). Overall Depth - inches (mm): 55.58 (1412). Overall Height - inches (mm): 56.43 (1433).

Approximate Net Weight - lbs. (kg): 944 (428).

** NOTE TO SPECIFIER ** Delete voltage options not required.

Electrical: Voltage: 220/3/50-60:  Running Amps: 7. Fuse (Amps): FRN15.  Circuit Breaker (Amps): 15.

Electrical: Voltage: 208,240/1/60.  Running Amps:  14, 12.  Fuse (Amps): FRN20.  Circuit Breaker (Amps):  20.

Electrical: Voltage: 208,240/3/60. Running Amps: 8,7. Fuse (Amps): FRN15. Circuit Breaker (Amps): 15.

Electrical: Voltage: 380/3/50-60: Running Amps: 4.5. Fuse (Amps): FRS15. Circuit Breaker (Amps): 15.

Electrical: Voltage: 415/3/50: Running Amps: 4.5. Fuse (Amps): FRS15. Circuit Breaker (Amps): 15.

Electrical: Voltage: 480/3/60: Running Amps: 3.5. Fuse (Amps): FRS15. Circuit Breaker (Amps): 15.