554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

February 28, 2023

Mr. Joe France

Building Safety Permits

P.O. Box 828

Slidell, La. 70459

Re:

3160 E Terrace Ave

Slidell, La. 70458

Dear Mr. France

Special K Construction requested that Dammon Engineering inspect the pilings for the raised home due to the chainsaw cuts being too deep for the 2x12 rim board’s themselves.

The repairs shall be;

- Install a 36” Simpson Strong Tie strap from the 2x12 floor joist to the piling past the cut, down to the good piling for a total of 32 pilings.

- Tighten all bolts on the pilings.

- Clean all the debris from under the home and slope the grade to the outside of home.

This report does not cover any other aspects of the building such as framing, plumbing, electrical, gas, mechanical egress, general arrangement, site drainage, etc., and assumes that ground conditions are substantially the same as is under the residence.

Sincerely,

Brian A Mistich, P.E.

La. License No.  0030187