Waveland Landscaping Co., 509 US-90, Waveland, MS 39576

Wind speed Calculations Result for a sign in Category 4

130-156 mph 113-136 kt 209-251 km/h

Waveland has a extreme Wind Factor™ risk based on the projected likelihood and speed of hurricane, tornado, or severe storm winds impacting it.  It is most at risk from hurricanes. Average maximum wind speeds in Waveland are higher now than they were 30 years ago.

| In addition to damaging properties, severe wind events can knock down trees or scatter debris that can cause harm to anyone outside during an event, or cut off access to utilities, emergency services, transportation, and may impact the overall economic well-being of an area. |  |
| --- | --- |

Recommended diameter of the subsurface concrete support Assuming Soil Pressure limit at 500 and 1000 psf and Sign support of 2’ and 3’ Diameter.

Depth of Sign Support in Soil

Soil Support                              1000 psf        500 psf

-----------------------------------------------------------------

2’ Diameter Support                   9.5’                     13’

3’ Diameter Support                   7.5’                  10.5’

-----------------------------------------------------------------

Included is the 8’ X 8’ X 1’ surface pad.