554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Scope Of Work

Slidell - Building 12 and 13 - Garage Repairs - 18 locations:
Frame out the opening at the back of each garage with PT Wood. 4’ x 4’ area
Install 1 half sheet of plywood
Install vapor barrier
Install 1 sheet sheet of Hardie Wall Panel 4’x4’.
Install 12’ LF of Trim
Caulk, Prime and paint the new wall area - 4’x4’
Cut out 4 holes on the side wall of the garage - approx 18” x 24”.
Install 4 new wall vents with wire mesh backing - approx 18” x 24”