6.6 For Single-Glazed Laminated Glass (LG) Constructed

With a PVB Interlayer Simply Supported Continuously Along

Four Sides Where In-Service Laminated Glass (LG) Temperatures

At The Design Load Do Not Exceed 50°C (122°F):

6.6.1 Determine the NFL from the appropriate chart (the

upper charts of Figs A1.28–A1.34) for the designated glass

thickness.

6.6.2 Determine the GTF for the appropriate glass type, load

duration (short or long) from Table 1.

6.6.3 Multiply NFL by GTF to get the LR of the laminated

lite.

6.6.4 Determine the approximate maximum lateral (center

of glass) deflection from the appropriate chart (the lower charts

of Figs. A1.28–A1.34) for the designated glass thickness, size,

and design load. If the maximum lateral deflection falls outside

the charts in Annex A1, then use the procedures outlined

inAppendix X1.

-------------------------------------------------------------------------------

TABLE 1 Glass Type Factors (GTF) for a Single Lite of Monolithic

or Laminated Glass (LG)

GTF

Glass Type          Short Duration Load (3 s)     Long Duration Load (30days)

AN                                     1.0                                           0.43

HS                                     2.0                                           1.3

FT                                      4.0                                           3.0

7. Report

7.1 Report the following information:

7.1.1 Date of calculation,

7.1.2 The specified design load and duration, the short

dimension of the glass, the long dimension of the glass, the

glass type(s) and thickness(es), the GTF(s), the LS factors (for

IG), the factored LR and the approximate lateral deflection, the

glass edge support conditions, and

7.1.3 A statement that the procedure followed was in

accordance with this practice or a full description of any

deviations.