ASTM E 1300 GLASS LOAD RESISTANCE REPORT

JOB DETAILS:

Project Name:		Woodmere Community Center

WINDOW GLASS DETAILS:  Lite C

Rectangular Dimensions:	Long:  42 in; Short: 35.75 in

Window Construction:	Double Glazed Insulating Glass Unit

Window Orientation:		Vertical

Outboard Lite		Inboard Lite

Glass Type:			Heat Strengthened	Heat Strengthened

Construction:			Monolithic		Laminated Glass

Nominal Thickness:		.25 in			0.509 in

Laminate Construction:				.25/0.09/.25

SHORT DURATION LOAD, RESISTANCE AND DEFLECTION DATA:

Design Load (<=3 sec.):		             1.91 kPa

Load Deflection:				.05 in

Maximum Allowable Glass Deflection:	L / 175

CONCLUSION:

Based on your design information, this glass configuration will resist the specified loading.

STATEMENT OF COMPLIANCE:

Procedures followed in determining the resistance of this window glass configuration are in accordance with ASTM E 1300-12a.

Disclaimer:

This software can be used to determine the load resistance of specified glass types exposed to uniform lateral loads of short or long duration subject to the following conditions:

- The glass is free of edge and surface damage and has been properly glazed in the opening in conformance with the manufacturers recommendations.

- The glass is supported on all four sides by a framing system sufficiently stiff to limit lateral deflection of the glass edge (not center-of-glass) less than to 1/175 of the glass edge length; Center of glass deflection in excess of 19-mm (0.75-in.) is a design issue and does not affect glass strength;

- The laminated glass factors for short term loads are representative of roof temperature data to which the glass is exposed.

For other limiting conditions that may apply, refer to Section 5 of ASTM E1300 and local building codes.

Prepared by : 	__________________________	Date:_____________________

Dammon Engineering, Inc